NUCLEAR MEDICINE SKIRT VEST LEAD-FREE APRON TECHNICAL SPECIFICATION

The apron must have lead-free protection against radiation equivalent to 1 mm on the front and 0.50 on the back. An analysis report regarding the lead-free material used in the product must be submitted.

Aprons should be two-piece, skirt and vest. Overlapping parts should be glued on the front with a velcro system. In order for the apron to be easy to use, there should be a vedifo system on the skirt part designed to provide waist support to prevent slipping. If desired, a waist support belt should be placed on the vest. There should be side pockets on the skirt.

For ergonomic use and long life of the apron, the inner fabric covering and fabric color of the apron and the outer fabric covering and color should be different. In accordance with safety standards, the gown must have a dosimeter pocket on the front. Upon the request of the institution and the user, there should be a dosimeter pocket on the inside of the gown.

The size of the apron should be chosen by the users and should fit the users' bodies. In this regard, the product provider company should supply products in sizes xs – s –m-l-xl-xxl-xxxl.

The apron label must contain manufacturer information, vendor information, size information, production date, lot number, barcode number, notified body number (CE) in accordance with the 2016/425 EU revision.

Lead-free aprons should have color options. The user should be able to choose a color according to the color options upon request. There should be a fabric option for lead aprons. The user should be able to request the product as oxfort, polyuran pu, hydrophobic or antibacterial upon request. A technical document must be submitted regarding the fabric used in the production of the product.

The x-ray protective lead apron must be produced and certified in accordance with the Personal Protective Equipment Regulation (Official Gazette No. 30761 dated 01 May 2019, new regulation (EU) 2016/425 It must.

There must be full approval and test reports for the production of the product. It should offer this when requested. At least 96% protection for the product must be documented with a test report.

The production conditions for the product must comply with ISO 9001:2015 standards and a document must be submitted proving that it complies with these standards.

There must be an ISO 45001: 2018 certificate for the production of radiation protective products, which is internationally valid, from an accredited institution for the production of the product.

It should be easy to use and fit around the body. The apron must have straps for easy hanging.

Lead-free aprons should be numbered with embroidery. Velcro name can be added if desired.

Lead-free aprons should be numbered with embroidery. Velcro name can be added if desired.

The fabric used must be waterproof, easy to wipe for long-term use and hygiene. The piping of the apron must be waterproof.

The manufacturer must provide a 2 (two) year warranty against production defects of the aprons.