LEAD Apron MUST PROVIDE PROTECTION AGAINST RADIATION EQUIVALENT TO 0.50 mm Pb ON THE FRONT AND 0.25 mm Pb LEAD ON THE BACK.

LEAD Apron MUST PROVIDE PROTECTION AGAINST RADIATION EQUIVALENT TO 0.50 mm Pb ON THE FRONT AND 0.25 mm Pb LEAD ON THE BACK.

Aprons should be two-piece, skirt and vest. Overlapping parts should be glued on the front with a velcro system. In order for the apron to be easy to use, there should be a vedifo system on the skirt part designed to provide waist support to prevent slipping. If desired, a waist support belt should be placed on the vest. There should be side pockets on the skirt.

The apron label must contain manufacturer information, vendor information, size information, production date, lot number, barcode number, notified body number (CE) in accordance with the 2016/425 EU revision.

The size of the apron should be chosen by the users and should fit the users' bodies. In this regard, the product provider company should supply products in sizes xs – s –m-l-xl-xxl-xxxl.

The apron label must contain manufacturer information, vendor information, size information, production date, lot number, barcode number, notified body number (CE) in accordance with the 2016/425 EU revision.

There must be a test report of the raw material manufacturer for the protective material. The company that produces the lead apron must submit a test report regarding the testing of the products, and a test report regarding the protective raw material must also be submitted from the raw material manufacturer.

In order to prevent counterfeit production of imported products, to prevent products that are tried to be imported by being licensed abroad and then produced here, products that are produced illegally in Türkiye by pretending to be German goods or goods of another country, and products sold under other names, if an imported product is to be supplied, an import declaration regarding the purchase must be submitted. There must be an acceptable period between the import date of the import declaration and the sale date of the imported product.

Lead aprons should have color options. The user should be able to choose a color according to the color options upon request. There should be a fabric option for lead aprons. The user should be able to request the product as oxfort, polyuran pu, hydrophobic or antibacterial upon request. A technical document must be submitted regarding the fabric used in the production of the product.

The x-ray protective lead apron must be produced and certified in accordance with the Personal Protective Equipment Regulation (official gazette dated 01 May 2019 and numbered 30761, new regulation (EU) 2016/425

In accordance with old standards for products, article 11 a article 11 b certificate should not be submitted. These documents have become obsolete and documents must be submitted for the sale of products certified according to the new revision.

There must be full approval and test reports for the production of the product. It should offer this when requested. A report showing at least 96% protection of the product must be submitted with the product.

The production conditions for the product must comply with ISO 9001:2015 standards and a document must be submitted proving that it complies with these standards.

There must be an ISO 45001: 2018 certificate for the production of radiation protective products, which is internationally valid, from an accredited institution for the production of the product.

The manufacturer must have ISO 13485:2016 quality management system certificate.

The manufacturer must have ISO 14001:2015 certification.

The seller must provide a work experience document or reference regarding previous work he has done regarding the products, if requested by the institution.

It should be easy to use and fit around the body. The apron must have straps for easy hanging.

The fabric used must be waterproof, easy to wipe for long-term use and hygiene. The piping of the apron must be waterproof.

The manufacturer must provide a 2 (two) year warranty against production defects of lead aprons.